1. Answer :

Pipelining is designed differently depending on purposes of hardware optimization. Pipelining are designed in 2 ways : RISC and CISC. These 2 are different sets of instructions for a computer that designed and executed in different ways.

RISC(Reduced Instructions Set for a Computer) : This focuses on optimizing the instruction pipeline by minimazing complexity and enabling fast execution. The instrucrions are usually simple and small tasks. Each instruction is executed in a single clock cycle while each instruction performs a single task. Approximately every task is getting executed for the same amount of time.

Also RISC uses a fixed-length of instructions, this means the system takes roughly the same amount of size instructions to execute. This allows for the processor not to worry about variable lengths and makes the process predictable, makes it easier to continue the process.

CISC(Complex Instructions Set for a Computer) - This system uses multiple instructions with varying lengths and types. 1 instruction can perform multiple tasks and an individual instruction can take multiple clock cycles to perform.

CISC can perform multiple varying operations, access memory or include different operands. In CISC, to perform more robust calculations, the length is varying in instructions, CPU takes them after decoding and divides them into micro-operands(uOps) for the pipeline. This can cause slowing down sadly.

Unlikely RISC or as we learned before, the pipelining can be more complex. CICS can include different number of stages for instructions whike in RICS, it is 5 or at most 7 all the time. But CISC can surpass this with way more complexity(10-20 stages in some cases)

CISC and RISC systems are used widely in the industry depending on their needs. RISC uses simples instructions which takes less power to process. This means more energy efficient, less heat and greater heat distribution for roughly repetitive instruction set and processing schedule. That's why they are used in Laptops, Phones, Tables.

CPUs with RISC system are called ARM CPUs. There are ARM x64 and ARM x86(or x32) CPUs in the market. Even though RISC provides great efficiencies, the heating of these devices are quite high. Laptops can easily reach 100 degree of heating on heavy processes!. But it is reasonable when we consider, the tight space the components are placed and non to only a pair of fans for cooling are there. But this heating do decrease the lifetime of the device. That’s why Laptops are worse in longevity comparing with PCs. PCs use CISC but that’s not because CISC better in heat consumption, in fact they are worse, higher degrees always being avoided. The main reason is that the PC system has a better cooling system.

CPUs with CISC system are used for PCs and more powerful devices. CISC processes less amount of instructions meaning more performance but because of the complexity of pipelining and instructions, it creates more heat and uses more power. Also varying and different pipeline and instructions creates less normal heat distribution. That's why PCs and more powerful machines like Supercomputers need heavy cooling systems. Also CISC is not build for working in high degrees. For example, while a laptop can work normally in 100 degrees, 100 degrees in a PC is an alarming value of heat. Most PCs turn themselves of when they hit 70 degrees. Intel and AMD are 2 leading CISC x64 and CISC x32(or x86) CPU manufacturers.

(I learned the information above since Highschool, Here are some books that I rememeber : Informatika Bələdçisi, Magistr kitabları)

2. Answer :

4D216=(4×162+13×161+2×160)=123410

4D216→116+216+416+816= 1516 , If we represent the numbers in Binary,

116→00012

216→00102

416→01002

816→10002 , thus, 416→01002, D16→11012, 216→00102

and finally 4D216→10011010010 2

3. Answer :

A CPU(Central Processing Unit) is like a plane. It takes mostly passengers and a fair amount of their luggage and baggages. It can not take tons of tons of weight like a cruise ship but a cruise ship is significantly slower than a plane. CPU is also like this, CPU has its processing cores but they are not much. Modern CPUs can have cores from 2 to 16. But what makes a CPU special is its processing speed. CPU can process very complicated calculations in incrediblely small amount of times.

For comparison again, a GPU(Graphical Processing Unit) is a cruise ship. A GPU has thousands of cores and can process way more calculations than a CPU. But a GPU is calculating individual calculations slower than a CPU, even though, it can take large amount of calculations in its cores. What makes a GPU special is the ability of its parallel computing. A GPU calculates trillions of trillions of Matrix calculations which are far more simple than CPU calculations, maybe slower in time but parallely way more efficient for large calculation processes.

A CPU could make this calculations incredibly fast but how? CPUs use a technic called Pipelining for this. At its core, pipelining is a method that allows a CPU to work on multiple instructions at once by dividing the work into different stages, much like an assembly line. Without pipelining, a CPU would execute one instruction at a time by finishing all steps of that instruction before starting the next one. With pipelining, multiple instructions can be in different stages of execution simultaneously.

In simpler terms, pipelining breaks down the instruction execution process into smaller steps (called stages), so the CPU can handle parts of different instructions at the same time, leading to faster overall performance.

Pipelining breaks the process into 5 stages :

  - Fetch : While a programming working in the memory, the program sends code, programming codes which are instructions for our computer. The process of fetching instructions from memory and sending them to cpu to process is called Fetching.

  - Decode : The instrucrions are transferred into machine code in this step. A normal code is written in multi lines for the user to understand, so the compiler breaks down it to 1 line and decodes it into machine code of 1s and 0s to be executed.

  - Execute : In the Execution process, the code that needs to be executed are transferred in between 2 level cache memories and CPU, CPU receives them and does the necessary calculations with using logical circuits of tranzistors in its cores.

  - Memory Access(if needed) : Some programs reading from or writing to memory.

  - Write back : The result of the execution is written back to the register or memory.

Most programs aren't in plain code. Some of them are getting compiled into Assembly, a nearer code to machine code or directly binary to not lose time on compiling.

What makes pipelining important is that each stage takes 1 clock cycle because they are independent. This means an instruction can be broken down into pieces and which part and when and where can be executed parallely or not as how CPU wants. This gives the CPU a “great flexibility”.

The main advantage of pipelining is throughout the number of instructions that can be completed in a given period. However, pipelining does not reduce the execution time of a single instruction. It only increases the number of instructions the CPU can handle at once.

The performance of a pipelined CPU can be evaluated using CPI (Cycles Per Instruction). In an ideal pipeline with no stalls, the CPI approaches 1, meaning one instruction is completed per clock cycle.

Pipelining increases efficiency but it will bring some problems and risks. There are three types of hazards:

- Data Hazards occur when instructions depend on the results of previous instructions. For example, if instruction B needs the result of instruction A, but instruction A hasn't completed yet, instruction B has to wait.

- Control Hazards happen when the CPU doesn’t know which instruction to fetch next, usually due to branches (like if statements or loops). If the branch decision hasn't been made, the CPU might fetch the wrong instruction.

- Structural Hazards occur when hardware resources are not sufficient to execute multiple instructions simultaneously. For example, if two instructions need to access memory at the same time and there is only one memory access port.

(I learned the information above since Highschool, Here are some books that I rememeber : Informatika Bələdçisi, Magistr kitabları)

4. Answer :

HDD(Hard Disk Drive) is a mechanical storage device to store data in a computer. It consists of 2 parts : The Hard Disk where the data is written magnetically, The Needle which is a needle with a very tiny tip and just a few nanometers above the disk. The data is written with dividing the disk’s surface into very tiny parts that has different magnetic poles. The data is not directly written as 1 North pole and 0 South pole but the needle goes through the poles and if the poles changes from North to South, then it is readden as 1, 0 otherwise.

The disk spins very fast, about thousands of spins per minute and the needle moves very fast, dozens of times a second. This reads and writes the data very fast. Additionally, there is not only 1 disk and needle but a few disks and needles places on top of eachother which multiplies the amount of data that can be stored in the device.

Even though, the process is very fast, it is not enough. An HDD boots a computer in about 1 minute and 41 seconds which is not great and the HDD that does this is a good HDD. Now we will move to SSDs. Why SSDs is the superior version in here? That starts from the architecture.

SSD(Solid State Drive) works very differently in comparison to HDDs. It stores the data in huge electrical chunks. We have billions of small units where the electrons are trapped in each unit, if the unit contains the electrons which will be detected as a “certain level” of electricity, it is 1, otherwise 0. But here, I want to clarify that, noise and distortion is always a factor, you will never get a clean 1v electricity or 0v electricity because of the noise distorts the data during the reading, writing or storing. To solve this, a certain interval of electricity which is usually between 5-3.5v is considered as enough electricity for us to assume that data is stored in the unit. If the electricity is below 1.5v, then it is enough for us to consider that the unit is 0 bits in data. In order to control all these massive chunks of units, the SSD consists of various amount of parts where each part is controlled from different pins(wires) on the SSD.

Now we exactly(not really but I also added a more in depth explanation later) know how a SSD works, and I think the question “Why SSDs are faster?” is out of the question. Simply, it is digital, not mechanical. We don’t need a disk to spin in relatively slow speeds comparing to an SSD’s speed, SSD works on electrical flows which is as fast as speed of light and the process of writing and reading is exceptionally faster than an HDD. In comparison. Modern SSDs boot a computer in under 20 seconds!

But we are here to also discuss the disadvantages. An SSD is very much more expensive than an HDD due to its complex and nanometric level architecture which is very costy and time consuming to produce. The reason why Servers like Google, Youtube and so on, still use HDDs, they are fast enough and is cheap to store all those gigantic databases of theirs.

Needle

Hard Disk

Solid State Drive

(I learned the information above since Highschool, Here are some books that I rememeber : Informatika Bələdçisi, Magistr kitabları)

(I also watched “Branch Academy” on Youtube, they have videos about this)

Optional(more in depth parts that I know and wanted to add) : I also want to go more in depth in here, yes, we know that the data is stored by trapping electrons in units but how exactly this happens, we can not simply let electrons to move to certain units, if we do like that, we can not store electrons because they will escape and we will lose data. In this case, we can make the borders of the unit high and thick enough for electrons to now pass. Pay attention, I also said it needs to be “thick enough” Why? Let’s continue and we’ll see.

In here, we will have a problem again, how we will let electrons to pass the border in order to write data? We will answer this question with a very interesting concept called Quantum tunneling. Electrons aren’t like a ball or small particle, they are more like a cloud, a cloud of probabilities which includes all the possible locations of the electron that can be. In this case, the electron is in a state called superposition, a state where a thing can be in multiple states at the same time. In this case, we will attract the electrons with placing a positively charged flow in the other side of the border and electrons will be attracted and pass through the wall with quantum tunneling and they will be trapped in the unit. This technology allows us to store electrons in a unit for up to 5-10 years!.

(I learned the information above from “Branch Academy” on Youtube)

5. Answer :

I wrote the answer in a NotePad File, If I write it here, It will have spacing problems.